THEREMIN

Main Board

All resistors are 1/2W 5% carbon

Pot 33K is A-law, 1W (wounded)

Capacitors

550 pF : (1%, 250V, ceramic)  associate 330 + 220

420 pF (id) 68 + 150

1 uF 10%, 160V DC, paper

10uF : 10%, 250V, chimical polarised

22uF : 10%, 40V, id

150 pF : air-variable

150 pF : air-adjust

tranformers

antic models used in old battery-receivers (1/3 and 1/5 ratio) : LF, iron core

electronic tubes

E424, E415, E438 : LF antic triodes (4V heating) can be replaced with (12AU7 ?)

AF2 : HF variable transconductance pentode, used as mixer (6DA6 ?)

C443 : final power pentode (6BQ5 ?)

B406 (osc) : replaced with 12AU7

B406 (var resistor) : replaced with 3A5 (the 2 triodes in parallel connection)

Coils (the most important !)

T1895 (beat oscillators transformer) :

On a 43 mm diameter linen tube (impregnated), 85 turns 25/100mm (insulated wire) (tube length :50 mm) wound start : grid, wound end : polarisation (4700 ohm, 1uF)

Then, insulation with adhesive tape (7/10 mm thick)

Finally, 85 turns, in opposite wound, like 1st winding. Wound start : plate, wound end : +HV

Characteristics measurement : the 2 sections in serie connection, L=1.3mH and Cr=50 pF

T1905 (vol oscillator) :

Same tube, but : 37 turns 6/10 mm insulated wire (grid winding)

Insulation : 48 mm diameter linen tube (stopped with little pieces of linen or plastic)

Plate winding : 37 turns, in opposite wound.

Connections : identical at T1895

Measurement : L=235uH, Cr=40 pF

T1899 (pitch control coil)

One solenoid coil and inside, one concentrated coil. These coils is in serie connection

Solenoid :

Linen tube, 83 mm diameter, length : 305 mm

Wound : start at the top (> pitch antenna) ; 1500 turns 16/100 mm (wound length : 270mm)

Measurement : L=46mH, Cr=3.9pF, R=320 ohm

Concentrated coil :

Plastique tube, 9.5mm diameter, length 21mm + core

Wound : 1400 turns 16/100

Measurement : 8.8mH (core off), … 28 mH (core at central position) / Cr=50 pF/ R=45 ohms

(refer drawing for concentrated coil positionning in solenoid coil and for connection)

T1907 (volume control coil)

Solenoid :

Linen tube, 83 mm diameter, length : 170 mm

Start wounding at the top (> volume antenna) ; 750 turns 16/100 (135 mm length)

Measurement :L=19 mH, Cr=6pF, R=160 ohm)

Pick-up coil :

Linen tube, 94 mm diameter, length : 20 mm

25 turns, 6/10 mm (16mm length)

when you slide this pick-up coil on the solenoid coil, you can adjust min volume. (start position : at the bottom) : refer drawing

# Power Unit

Nothing particular,

Power transformer (40 ..50 VA), for you 120 V primary voltage !

Instead 4V, use 6.3V for heating !

Replace fixed polarisation by automatic polarisation in 6BQ5 cathod return ! (and connect 6BQ5 cathod and 12AU7 volume oscillator together)

Use loud-speaker transformer in respect of 6BQ5 impedance !

See schematics